⚡

Energy-Efficient CPU Scheduling Algorithm

CSE-316 — Operating Systems  |  CA2 Project Report

Submitted By

| Student Name | SAI HITESH YADAV |
| --- | --- |
| Roll Number | 52 ¦ R224EIB52 |
| Registration No | 12410928 |
| Course | B.Tech — CSE |
| Subject | CSE-316: Operating Systems |
| Assignment | CA2 — Project Report |
| Session | 2024–25 |

# 1. Project Overview

Modern computing devices — especially smartphones, IoT sensors, wearables, and embedded systems — operate under tight power budgets. Traditional CPU schedulers optimize for throughput or response time but ignore energy consumption, leading to rapid battery drain and thermal issues.

This project designs and implements an Energy-Efficient CPU Scheduling Algorithm that dynamically adapts processor frequency and voltage through Dynamic Voltage and Frequency Scaling (DVFS), combined with a hybrid scheduling policy merging Earliest Deadline First (EDF) for real-time tasks and Power-Aware Round Robin for general processes.

Goals

- Minimize total CPU energy consumption during task execution.

- Maintain acceptable performance: low waiting time and turnaround time.

- Support real-time workloads using deadline-aware scheduling.

- Provide an interactive GUI to visualize scheduling and energy metrics.

Expected Outcomes

- A working Python-based scheduler simulator with a full GUI.

- Gantt chart showing process execution and frequency levels used.

- Energy comparison between DVFS approach and full-frequency baseline.

- Per-process statistics: waiting time, turnaround time, energy consumed.

Scope

The project is implemented as a desktop simulation tool that accurately models energy and scheduling behavior. Normalized energy units consistent with published DVFS research models are used.

# 2. Module-Wise Breakdown

The project is divided into three well-defined modules:

## Module 1 — Core Scheduling Engine

- Process data structure: PID, name, arrival time, burst time, priority, deadline, power class.

- DVFSModel class — models four frequency levels (25%, 50%, 75%, 100%) with power values. Picks the lowest frequency that meets the deadline.

- EnergyEfficientScheduler — EDF urgency-based selection, adaptive Round Robin quantum, DVFS assignment, and CPU sleep state when idle.

## Module 2 — GUI Frontend (Tkinter)

- Process Input Panel — name, arrival, burst, priority, deadline, power class, quantum fields.

- Process Table — displays all added processes with delete support.

- Random Process Generator — auto-generates 5–8 processes for quick testing.

- Results Notebook — three tabs: Gantt Chart, Energy Analysis, Process Stats.

## Module 3 — Data Visualization (Matplotlib)

- Gantt Chart — horizontal bars per process; border color indicates CPU frequency.

- Energy Bar Chart — baseline vs DVFS energy per process side-by-side.

- Pie Chart — CPU time distribution across frequency levels and sleep states.

- Stats Table — per-process metrics in a scrollable TTK Treeview.

# 3. Functionalities

## Module 1 — Core Scheduler

- Sort and parse processes by arrival time.

- DVFS frequency selection: slack = deadline − time; ratio = remaining/slack; pick lowest satisfying frequency.

- Urgency-based selection: urgency = remaining / slack; highest urgency executes next (EDF variant).

- Adaptive quantum: high-priority × 1.5, low-priority × 0.75 of base quantum.

- CPU sleep when no process is ready; near-zero energy until next arrival.

- Per-process energy tracking: energy = power_factor[freq] × exec_time.

## Module 2 — GUI

- Add, delete, and clear processes interactively.

- Random process generation for quick demo.

- Configure base time quantum.

- Trigger scheduler run and view tabbed results.

## Module 3 — Visualization

- Gantt chart with color-coded frequency per process slice.

- Bar chart: baseline vs DVFS energy per process.

- Pie chart: time split across frequency levels and sleep.

- Tabular stats: waiting time, turnaround time, energy, average frequency.

# 4. Technology Used

| Tool / Library | Purpose |
| --- | --- |
| Python 3.x | Core implementation language |
| Tkinter | GUI framework (built-in with Python) |
| Matplotlib | Gantt chart, bar chart, pie chart visualization |
| collections.deque | Efficient ready-queue management |
| math, random | Math utilities and random process generation |
| GitHub | Version control & revision tracking |

Other Tools

- GitHub — version control, commit history, branching.

- VS Code — development IDE.

# 5. Flow Diagram

The diagram below illustrates the complete execution flow of the Energy-Efficient CPU Scheduler, from process loading through DVFS-based scheduling to final output generation.

START

▼

Step 1: Load & Sort Processes by Arrival Time

▼

Step 2: Initialize → current_time = 0, ready_queue = empty

▼

Step 3: Scan for Arrived Processes (arrival ≤ current_time) Add them to Ready Queue

▼

◆ Is Ready Queue EMPTY? ◆

| YES → Any future processes remaining? YES: CPU enters SLEEP state current_time = next arrival time Record sleep energy (≈ 0) NO: All done → go to OUTPUT |  | NO → Proceed to Process Selection |
| --- | --- | --- |

▼

Step 4: Select Process Compute Urgency = remaining_burst / (deadline − current_time) Pick process with HIGHEST urgency (EDF variant)

▼

Step 5: DVFS Frequency Selection slack = deadline − current_time ratio = remaining / slack ratio ≤ 0.25 → freq = 25% (most efficient) ratio ≤ 0.50 → freq = 50% ratio ≤ 0.75 → freq = 75% ratio > 0.75 → freq = 100% (full power)

▼

Step 6: Compute Adaptive Time Quantum Priority 3 (High) → quantum × 1.5 Priority 2 (Normal) → quantum × 1.0 Priority 1 (Low) → quantum × 0.75

▼

Step 7: Execute Process Slice exec_units = min(remaining_burst, quantum) exec_time = exec_units / frequency energy = power_factor[freq] × exec_time current_time += exec_time Update: process.remaining, process.energy_consumed

▼

◆ Is process.remaining == 0? ◆

| YES → Process COMPLETE Record: finish_time = current_time turnaround = finish − arrival waiting = turnaround − burst |  | NO → Process NOT done Re-enqueue process into Ready Queue (preempted) |
| --- | --- | --- |

▼

◆ Any processes remaining (ready or future)? ◆

| YES → Go back to Step 3 (Scan for new arrivals & continue scheduling) |  | NO → All processes complete → Proceed to Output |
| --- | --- | --- |

▼

OUTPUT • Gantt Chart (frequency color-coded) • Energy Bar Chart (DVFS vs Baseline) • CPU Time Pie Chart (freq distribution) • Process Stats Table (wait, turnaround, energy)

▼

END

Flow Diagram Legend

| Symbol | Meaning |
| --- | --- |
| Blue box | Process step / action |
| Yellow ◆ | Decision point (branch) |
| Green box | YES branch outcome |
| Pink box | NO branch outcome |
| ▼ Arrow | Flow direction |

# 6. Revision Tracking on GitHub

| Repository Name | energy-efficient-cpu-scheduler |
| --- | --- |
| GitHub Link | https://github.com/krishan-kumar/energy-efficient-cpu-scheduler |
| Branch Strategy | main (stable) + feature branches per module |
| Min. Revisions | 7+ commits with descriptive messages |

Suggested Commit History

| # | Commit Message | Branch |
| --- | --- | --- |
| 1 | Initial project structure and README | main |
| 2 | Add Process class and DVFSModel | feature/core-scheduler |
| 3 | Implement EDF urgency-based selection | feature/core-scheduler |
| 4 | Add adaptive quantum and sleep state | feature/core-scheduler |
| 5 | Merge core-scheduler into main | main |
| 6 | Build Tkinter GUI with process input panel | feature/gui |
| 7 | Integrate Matplotlib Gantt chart | feature/visualization |
| 8 | Add energy bar and pie charts | feature/visualization |
| 9 | Final integration, testing, and cleanup | main |

# 7. Conclusion and Future Scope

## Conclusion

This project successfully implements an Energy-Efficient CPU Scheduling Algorithm integrating DVFS, EDF-based process selection, adaptive time quantum, and CPU sleep state management into a cohesive simulator with a rich GUI.

The DVFS mechanism selects the minimum CPU frequency that allows tasks to meet their deadlines, trading slightly higher execution time for significantly lower power consumption. In typical test runs with mixed workloads, the scheduler achieves 30–60% energy savings compared to running all tasks at full frequency.

## Future Scope

- Integrate with real OS APIs (Linux cpufreq, Android PowerManager) for actual hardware DVFS control.

- Add machine-learning-based burst time prediction to improve frequency decisions.

- Extend to multi-core scheduling with per-core DVFS and load balancing.

- Add battery-level-aware scheduling (slow down when battery < 20%).

- Export scheduling reports as PDF.

# 8. References

- Silberschatz, A., Galvin, P. B., & Gagne, G. (2018). Operating System Concepts (10th ed.). Wiley.

- Weiser, M., Welch, B., Demers, A., & Shenker, S. (1994). Scheduling for Reduced CPU Energy. OSDI Proceedings.

- Pillai, P., & Shin, K. G. (2001). Real-time Dynamic Voltage Scaling for Low-Power Embedded Operating Systems. ACM SOSP.

- Liu, C. L., & Layland, J. W. (1973). Scheduling Algorithms for Multiprogramming in a Hard-Real-Time Environment. Journal of the ACM, 20(1), 46–61.

- Python Documentation — tkinter, collections, math. https://docs.python.org/3/

- Matplotlib Documentation. https://matplotlib.org/stable/

# Appendix A — AI-Generated Project Elaboration

The following breakdown was generated by an AI system (Claude by Anthropic) based on the problem statement:

Project Overview

The project targets energy-constrained environments such as mobile and embedded systems. Standard schedulers (FCFS, Round Robin, Priority) maximize throughput or minimize latency without considering power draw. This scheduler integrates power modeling directly into scheduling decisions.

Module-Wise Breakdown

- Module 1 (Scheduling Core): DVFS frequency model, urgency-based EDF selection, adaptive quantum, sleep state.

- Module 2 (GUI): Tkinter-based interface for process management and results display.

- Module 3 (Visualization): Matplotlib charts for Gantt, energy comparison, time distribution.

Key Functionalities

- Dynamic frequency scaling based on deadline slack ratio.

- Urgency metric = remaining_burst / time_slack for EDF-style selection.

- CPU sleep when idle; near-zero power draw recorded.

- Per-process energy accounting and overall energy saving % vs full-frequency baseline.

Technology Recommendations

- Python 3 — rapid prototyping, rich library ecosystem.

- Tkinter — zero-dependency GUI for desktop simulation.

- Matplotlib — charts embedded in Tkinter via FigureCanvasTkAgg.

Execution Plan

- Define Process and DVFSModel classes.

- Implement EnergyEfficientScheduler with DVFS + EDF logic.

- Build Tkinter GUI layout.

- Integrate Matplotlib charts into notebook tabs.

- Test with random and edge-case workloads.

- Document and push to GitHub with 7+ commits.

# Appendix B — Problem Statement

Project Title: Energy-Efficient CPU Scheduling Algorithm Problem Description: Develop a CPU scheduling algorithm that minimizes energy consumption without compromising performance. The algorithm should be suitable for mobile and embedded systems where power efficiency is critical.

# Appendix C — Complete Source Code

File: energy_cpu_scheduler/scheduler.py

"""

Energy-Efficient CPU Scheduling Algorithm

CSE-316 CA2 Project | Roll No: 35 | Suraj Rawat (R224EIA69)

Implements an Energy-Efficient CPU Scheduler that combines:

- Dynamic Voltage and Frequency Scaling (DVFS)

- Power-aware Round Robin with adaptive time quantum

- EDF (Earliest Deadline First) for real-time tasks

- Sleep/idle state management

"""

import tkinter as tk

from tkinter import ttk, messagebox

import random

import math

import time

import threading

import matplotlib

matplotlib.use('TkAgg')

import matplotlib.pyplot as plt

import matplotlib.patches as mpatches

from matplotlib.backends.backend_tkagg import FigureCanvasTkAgg

from collections import deque

import copy

# ─────────────────────────────────────────────

#  Core Data Structures

# ─────────────────────────────────────────────

class Process:

def __init__(self, pid, name, arrival, burst, priority=1, deadline=None, power_class='normal'):

self.pid = pid

self.name = name

self.arrival = arrival

self.burst = burst

self.remaining = burst

self.priority = priority          # 1=low, 2=medium, 3=high

self.deadline = deadline if deadline else arrival + burst * 3

self.power_class = power_class    # 'low', 'normal', 'high'

self.start_time = None

self.finish_time = None

self.waiting_time = 0

self.turnaround_time = 0

self.energy_consumed = 0.0

self.frequency_used = []          # track freq per time unit

def __repr__(self):

return f"P{self.pid}({self.name})"

class CPUState:

ACTIVE   = 'active'

IDLE     = 'idle'

SLEEP    = 'sleep'

class DVFSModel:

"""

Simplified DVFS: voltage & power scale roughly as freq^2.

Frequencies (normalized): 0.25, 0.50, 0.75, 1.0

"""

LEVELS = [0.25, 0.50, 0.75, 1.0]

# Power ~ V^2 * f, V scales ~linearly with f  =>  power ~ f^3 roughly

POWER  = {0.25: 0.06, 0.50: 0.20, 0.75: 0.50, 1.0: 1.00}  # relative units

@staticmethod

def choose_freq(remaining_time, deadline, current_time):

"""Pick the lowest frequency that can finish before deadline."""

slack = deadline - current_time

if slack <= 0:

return 1.0   # must rush

ratio = remaining_time / slack

if ratio <= 0.25:

return 0.25

elif ratio <= 0.50:

return 0.50

elif ratio <= 0.75:

return 0.75

else:

return 1.0

@staticmethod

def exec_time_at_freq(burst_units, freq):

"""Real-time units at given frequency (lower freq = slower)."""

return burst_units / freq

@staticmethod

def energy(burst_units, freq):

"""Energy = power * time."""

t = DVFSModel.exec_time_at_freq(burst_units, freq)

p = DVFSModel.POWER[freq]

return p * t

class EnergyEfficientScheduler:

"""

Hybrid scheduler:

1. Real-time tasks -> EDF with DVFS

2. Normal tasks   -> Power-aware Round Robin (adaptive quantum)

3. CPU idles when queue empty -> sleep state (0 energy draw)

"""

def __init__(self, processes, base_quantum=4):

self.processes   = sorted(copy.deepcopy(processes), key=lambda p: p.arrival)

self.base_quantum = base_quantum

self.timeline    = []   # list of (pid, start, end, freq, state)

self.current_time = 0

self.cpu_state   = CPUState.IDLE

self.total_energy = 0.0

self.idle_energy  = 0.005  # per unit time in idle

self.sleep_energy = 0.001  # per unit time in sleep

def _adaptive_quantum(self, process):

"""Scale quantum: high-priority / deadline-critical tasks get longer slice."""

q = self.base_quantum

if process.priority == 3:

q = int(q * 1.5)

elif process.priority == 1:

q = max(2, int(q * 0.75))

return q

def run(self):

remaining  = self.processes[:]

ready_q    = deque()

t = 0

while remaining or ready_q:

# Enqueue arrived processes

arrived = [p for p in remaining if p.arrival <= t]

for p in arrived:

ready_q.append(p)

remaining.remove(p)

if not ready_q:

# CPU sleep until next arrival

next_arr = min(p.arrival for p in remaining) if remaining else t

sleep_dur = next_arr - t

self.timeline.append(('SLEEP', t, next_arr, 0, CPUState.SLEEP))

self.total_energy += self.sleep_energy * sleep_dur

t = next_arr

continue

# Pick next process (EDF-aware: choose closest deadline first among ready)

ready_list = list(ready_q)

# Urgency = remaining / slack

def urgency(p):

slack = p.deadline - t

if slack <= 0:

return float('inf')

return p.remaining / slack

ready_list.sort(key=urgency, reverse=True)

proc = ready_list[0]

ready_q = deque([p for p in ready_q if p != proc])

# Choose frequency via DVFS

freq = DVFSModel.choose_freq(proc.remaining, proc.deadline, t)

quantum = self._adaptive_quantum(proc)

# How many real-time units this quantum takes at chosen freq

real_units = min(proc.remaining, quantum)

exec_t = math.ceil(DVFSModel.exec_time_at_freq(real_units, freq))

if proc.start_time is None:

proc.start_time = t

# Execute

start = t

t += exec_t

proc.remaining -= real_units

energy = DVFSModel.energy(real_units, freq)

proc.energy_consumed += energy

self.total_energy += energy

proc.frequency_used.append(freq)

self.timeline.append((proc.pid, start, t, freq, CPUState.ACTIVE))

if proc.remaining <= 0:

proc.finish_time = t

proc.turnaround_time = t - proc.arrival

proc.waiting_time = proc.turnaround_time - proc.burst

else:

# Re-enqueue remaining

ready_q.appendleft(proc)

# Enqueue any newly arrived

arrived2 = [p for p in remaining if p.arrival <= t]

for p in arrived2:

ready_q.append(p)

remaining.remove(p)

# Baseline energy (all at max freq, no DVFS)

self.baseline_energy = sum(

DVFSModel.energy(p.burst, 1.0) for p in self.processes

)

return self

# ─────────────────────────────────────────────

#  GUI Application

# ─────────────────────────────────────────────

FREQ_COLORS = {

0.25: '#4CAF50',   # green  - very efficient

0.50: '#8BC34A',   # light green

0.75: '#FF9800',   # orange - moderate

1.0:  '#F44336',   # red    - full power

}

SLEEP_COLOR = '#90CAF9'

IDLE_COLOR  = '#EEEEEE'

PROCESS_COLORS = [

'#7E57C2','#42A5F5','#26A69A','#EC407A',

'#FF7043','#66BB6A','#FFCA28','#AB47BC',

'#26C6DA','#D4E157',

]

class App(tk.Tk):

def __init__(self):

super().__init__()

self.title("Energy-Efficient CPU Scheduler  |  CSE-316  |  Roll No: 35")

self.geometry("1280x820")

self.configure(bg='#1E1E2E')

self.resizable(True, True)

self.processes = []

self.pid_counter = 1

self.result = None

self._build_ui()

# ── UI Builder ────────────────────────────

def _build_ui(self):

top = tk.Frame(self, bg='#1E1E2E')

top.pack(fill='x', padx=12, pady=8)

tk.Label(top, text="⚡ Energy-Efficient CPU Scheduler",

font=('Helvetica', 18, 'bold'), fg='#CDD6F4', bg='#1E1E2E').pack(side='left')

tk.Label(top, text="Roll No: 35  |  Suraj Rawat  |  CSE-316",

font=('Helvetica', 10), fg='#A6ADC8', bg='#1E1E2E').pack(side='right')

# ── Input Panel ──

inp = tk.LabelFrame(self, text=" Add Process ", font=('Helvetica', 10, 'bold'),

fg='#CDD6F4', bg='#313244', bd=1, relief='groove')

inp.pack(fill='x', padx=12, pady=4)

fields = [

('Name',    'name_var',     'P1',   8),

('Arrival', 'arrival_var',  '0',    5),

('Burst',   'burst_var',    '5',    5),

('Priority (1-3)', 'prio_var', '2', 5),

('Deadline','deadline_var', '',     6),

]

self.name_var     = tk.StringVar(value='P1')

self.arrival_var  = tk.StringVar(value='0')

self.burst_var    = tk.StringVar(value='5')

self.prio_var     = tk.StringVar(value='2')

self.deadline_var = tk.StringVar(value='')

self.quantum_var  = tk.StringVar(value='4')

self.power_var    = tk.StringVar(value='normal')

for col, (label, var, default, w) in enumerate(fields):

tk.Label(inp, text=label+':', fg='#CDD6F4', bg='#313244',

font=('Helvetica',9)).grid(row=0, column=col*2, padx=4, pady=6, sticky='e')

tk.Entry(inp, textvariable=getattr(self, var), width=w,

bg='#45475A', fg='#CDD6F4', insertbackground='white').grid(

row=0, column=col*2+1, padx=4)

tk.Label(inp, text='Power Class:', fg='#CDD6F4', bg='#313244',

font=('Helvetica',9)).grid(row=0, column=10, padx=4)

ttk.Combobox(inp, textvariable=self.power_var, values=['low','normal','high'],

width=7, state='readonly').grid(row=0, column=11, padx=4)

tk.Label(inp, text='Quantum:', fg='#CDD6F4', bg='#313244',

font=('Helvetica',9)).grid(row=0, column=12, padx=4)

tk.Entry(inp, textvariable=self.quantum_var, width=4,

bg='#45475A', fg='#CDD6F4', insertbackground='white').grid(row=0, column=13, padx=4)

btn_f = tk.Frame(inp, bg='#313244')

btn_f.grid(row=0, column=14, padx=8)

tk.Button(btn_f, text='➕ Add', command=self._add_process,

bg='#89B4FA', fg='#1E1E2E', font=('Helvetica',9,'bold'),

relief='flat', padx=8).pack(side='left', padx=2)

tk.Button(btn_f, text='🎲 Random', command=self._random_processes,

bg='#A6E3A1', fg='#1E1E2E', font=('Helvetica',9,'bold'),

relief='flat', padx=8).pack(side='left', padx=2)

tk.Button(btn_f, text='🗑 Clear', command=self._clear,

bg='#F38BA8', fg='#1E1E2E', font=('Helvetica',9,'bold'),

relief='flat', padx=8).pack(side='left', padx=2)

# ── Process Table ──

tbl_frame = tk.Frame(self, bg='#1E1E2E')

tbl_frame.pack(fill='x', padx=12, pady=4)

cols = ('PID','Name','Arrival','Burst','Priority','Deadline','Power')

self.tree = ttk.Treeview(tbl_frame, columns=cols, show='headings', height=4)

style = ttk.Style()

style.theme_use('clam')

style.configure('Treeview', background='#313244', foreground='#CDD6F4',

fieldbackground='#313244', rowheight=22)

style.configure('Treeview.Heading', background='#45475A', foreground='#CDD6F4', font=('Helvetica',9,'bold'))

widths = [40,80,60,60,70,70,70]

for c, w in zip(cols, widths):

self.tree.heading(c, text=c)

self.tree.column(c, width=w, anchor='center')

self.tree.pack(side='left', fill='x', expand=True)

sb = ttk.Scrollbar(tbl_frame, orient='vertical', command=self.tree.yview)

self.tree.configure(yscrollcommand=sb.set)

sb.pack(side='left', fill='y')

self.tree.bind('<Delete>', lambda e: self._delete_selected())

# ── Run Button ──

run_f = tk.Frame(self, bg='#1E1E2E')

run_f.pack(pady=4)

tk.Button(run_f, text='▶  Run Energy-Efficient Scheduler', command=self._run,

bg='#CBA6F7', fg='#1E1E2E', font=('Helvetica',12,'bold'),

relief='flat', padx=20, pady=6).pack()

# ── Notebook for results ──

nb_frame = tk.Frame(self, bg='#1E1E2E')

nb_frame.pack(fill='both', expand=True, padx=12, pady=4)

style.configure('TNotebook', background='#1E1E2E', borderwidth=0)

style.configure('TNotebook.Tab', background='#313244', foreground='#CDD6F4',

padding=[10,4], font=('Helvetica',9,'bold'))

style.map('TNotebook.Tab', background=[('selected','#CBA6F7')],

foreground=[('selected','#1E1E2E')])

self.nb = ttk.Notebook(nb_frame)

self.nb.pack(fill='both', expand=True)

self.gantt_frame  = tk.Frame(self.nb, bg='#1E1E2E')

self.energy_frame = tk.Frame(self.nb, bg='#1E1E2E')

self.stats_frame  = tk.Frame(self.nb, bg='#1E1E2E')

self.nb.add(self.gantt_frame,  text=' 📊 Gantt Chart ')

self.nb.add(self.energy_frame, text=' ⚡ Energy Analysis ')

self.nb.add(self.stats_frame,  text=' 📋 Process Stats ')

# ── Process Management ─────────────────────

def _add_process(self):

try:

name     = self.name_var.get().strip() or f'P{self.pid_counter}'

arrival  = int(self.arrival_var.get())

burst    = int(self.burst_var.get())

priority = int(self.prio_var.get())

deadline = int(self.deadline_var.get()) if self.deadline_var.get() else None

power    = self.power_var.get()

assert 1 <= priority <= 3

assert burst > 0

except Exception:

messagebox.showerror("Input Error", "Check your inputs.\nPriority: 1-3, Burst > 0.")

return

p = Process(self.pid_counter, name, arrival, burst, priority, deadline, power)

self.processes.append(p)

self.tree.insert('', 'end', iid=str(self.pid_counter),

values=(p.pid, p.name, p.arrival, p.burst,

p.priority, p.deadline, p.power_class))

self.pid_counter += 1

# auto-increment name

try:

base = ''.join(filter(str.isalpha, name))

num  = int(''.join(filter(str.isdigit, name)) or '0') + 1

self.name_var.set(base + str(num))

except Exception:

pass

def _random_processes(self):

self._clear()

n = random.randint(5, 8)

t = 0

for i in range(n):

t += random.randint(0, 3)

burst = random.randint(2, 10)

prio  = random.randint(1, 3)

dl    = t + burst + random.randint(burst, burst*3)

pc    = random.choice(['low','normal','normal','high'])

self.name_var.set(f'P{i+1}')

self.arrival_var.set(str(t))

self.burst_var.set(str(burst))

self.prio_var.set(str(prio))

self.deadline_var.set(str(dl))

self.power_var.set(pc)

self._add_process()

def _delete_selected(self):

sel = self.tree.selection()

for iid in sel:

pid = int(iid)

self.processes = [p for p in self.processes if p.pid != pid]

self.tree.delete(iid)

def _clear(self):

self.processes = []

self.pid_counter = 1

for row in self.tree.get_children():

self.tree.delete(row)

for f in (self.gantt_frame, self.energy_frame, self.stats_frame):

for w in f.winfo_children():

w.destroy()

# ── Run Scheduler ─────────────────────────

def _run(self):

if not self.processes:

messagebox.showwarning("No Processes", "Add at least one process first.")

return

try:

q = int(self.quantum_var.get())

except Exception:

q = 4

sched = EnergyEfficientScheduler(self.processes, base_quantum=q)

sched.run()

self.result = sched

self._draw_gantt(sched)

self._draw_energy(sched)

self._draw_stats(sched)

self.nb.select(0)

# ── Gantt Chart ───────────────────────────

def _draw_gantt(self, sched):

for w in self.gantt_frame.winfo_children():

w.destroy()

pid_list = sorted(set(p.pid for p in sched.processes))

pid_map  = {pid: i for i, pid in enumerate(pid_list)}

proc_map = {p.pid: p for p in sched.processes}

fig, ax = plt.subplots(figsize=(13, max(3, len(pid_list)*0.7 + 1.5)))

fig.patch.set_facecolor('#1E1E2E')

ax.set_facecolor('#313244')

for entry in sched.timeline:

pid, start, end, freq, state = entry

if state == CPUState.SLEEP:

ax.barh(0.5, end-start, left=start, height=0.6,

color=SLEEP_COLOR, alpha=0.5, edgecolor='none')

ax.text((start+end)/2, 0.5, 'sleep', ha='center', va='center',

fontsize=7, color='#555')

continue

row = pid_map[pid] + 1

color = PROCESS_COLORS[pid % len(PROCESS_COLORS)]

bar = ax.barh(row, end-start, left=start, height=0.6,

color=color, alpha=0.85,

edgecolor=FREQ_COLORS[freq], linewidth=2)

ax.text((start+end)/2, row, f'{proc_map[pid].name}\n{int(freq*100)}%',

ha='center', va='center', fontsize=7, color='white', fontweight='bold')

yticks = [0.5] + list(range(1, len(pid_list)+1))

ylabels = ['CPU'] + [proc_map[pid].name for pid in pid_list]

ax.set_yticks(yticks)

ax.set_yticklabels(ylabels, color='#CDD6F4')

ax.set_xlabel('Time Units', color='#CDD6F4')

ax.set_title('Gantt Chart  (border color = CPU frequency)', color='#CDD6F4', fontsize=11)

ax.tick_params(colors='#CDD6F4')

for spine in ax.spines.values():

spine.set_edgecolor('#45475A')

legend = [mpatches.Patch(color=FREQ_COLORS[f], label=f'{int(f*100)}% freq') for f in DVFSModel.LEVELS]

legend.append(mpatches.Patch(color=SLEEP_COLOR, label='Sleep'))

ax.legend(handles=legend, loc='lower right', fontsize=8,

facecolor='#45475A', labelcolor='#CDD6F4', framealpha=0.8)

fig.tight_layout()

canvas = FigureCanvasTkAgg(fig, self.gantt_frame)

canvas.draw()

canvas.get_tk_widget().pack(fill='both', expand=True)

plt.close(fig)

# ── Energy Chart ──────────────────────────

def _draw_energy(self, sched):

for w in self.energy_frame.winfo_children():

w.destroy()

procs = [p for p in sched.processes if p.finish_time is not None]

if not procs:

return

fig, (ax1, ax2) = plt.subplots(1, 2, figsize=(13, 4))

fig.patch.set_facecolor('#1E1E2E')

for ax in (ax1, ax2):

ax.set_facecolor('#313244')

ax.tick_params(colors='#CDD6F4')

for spine in ax.spines.values():

spine.set_edgecolor('#45475A')

# Bar: per-process energy vs baseline

names    = [p.name for p in procs]

actual   = [p.energy_consumed for p in procs]

baseline = [DVFSModel.energy(p.burst, 1.0) for p in procs]

x = range(len(names))

ax1.bar([i-0.2 for i in x], baseline, 0.35, label='Baseline (full freq)',

color='#F38BA8', alpha=0.8)

ax1.bar([i+0.2 for i in x], actual,   0.35, label='DVFS actual',

color='#A6E3A1', alpha=0.8)

ax1.set_xticks(list(x)); ax1.set_xticklabels(names, color='#CDD6F4')

ax1.set_ylabel('Energy (relative units)', color='#CDD6F4')

ax1.set_title('Per-Process Energy: Baseline vs DVFS', color='#CDD6F4')

ax1.legend(facecolor='#45475A', labelcolor='#CDD6F4')

# Pie: freq distribution across all timeline entries

freq_time = {f: 0 for f in DVFSModel.LEVELS}

sleep_t   = 0

for entry in sched.timeline:

_, start, end, freq, state = entry

if state == CPUState.SLEEP:

sleep_t += end - start

else:

freq_time[freq] += end - start

labels = [f'{int(f*100)}% freq' for f in DVFSModel.LEVELS] + ['Sleep']

sizes  = [freq_time[f] for f in DVFSModel.LEVELS] + [sleep_t]

colors = [FREQ_COLORS[f] for f in DVFSModel.LEVELS] + [SLEEP_COLOR]

sizes_f = [s for s in sizes if s > 0]

labels_f= [l for l, s in zip(labels, sizes) if s > 0]

colors_f= [c for c, s in zip(colors, sizes) if s > 0]

ax2.pie(sizes_f, labels=labels_f, colors=colors_f, autopct='%1.1f%%',

textprops={'color':'#CDD6F4'}, startangle=90)

savings = (1 - sched.total_energy/sched.baseline_energy)*100 if sched.baseline_energy else 0

ax2.set_title(f'CPU Time Distribution\nEnergy Saved: {savings:.1f}%', color='#CDD6F4')

fig.tight_layout()

canvas = FigureCanvasTkAgg(fig, self.energy_frame)

canvas.draw()

canvas.get_tk_widget().pack(fill='both', expand=True)

plt.close(fig)

# ── Stats Table ───────────────────────────

def _draw_stats(self, sched):

for w in self.stats_frame.winfo_children():

w.destroy()

# Summary metrics

procs = [p for p in sched.processes if p.finish_time is not None]

avg_wait = sum(p.waiting_time for p in procs) / len(procs) if procs else 0

avg_ta   = sum(p.turnaround_time for p in procs) / len(procs) if procs else 0

savings  = (1 - sched.total_energy/sched.baseline_energy)*100 if sched.baseline_energy else 0

summary = tk.Frame(self.stats_frame, bg='#313244')

summary.pack(fill='x', padx=8, pady=6)

metrics = [

('Avg Waiting Time',    f'{avg_wait:.2f} units'),

('Avg Turnaround Time', f'{avg_ta:.2f} units'),

('Total Energy (DVFS)', f'{sched.total_energy:.3f} units'),

('Baseline Energy',     f'{sched.baseline_energy:.3f} units'),

('Energy Saved',        f'{savings:.1f}%'),

]

for i, (label, val) in enumerate(metrics):

tk.Label(summary, text=label+':', fg='#A6ADC8', bg='#313244',

font=('Helvetica',10)).grid(row=0, column=i*2, padx=10, pady=6, sticky='e')

tk.Label(summary, text=val, fg='#A6E3A1', bg='#313244',

font=('Helvetica',10,'bold')).grid(row=0, column=i*2+1, padx=4)

# Per-process table

cols = ('PID','Name','Arrival','Burst','Finish','Wait','Turnaround','Energy','Avg Freq')

style = ttk.Style()

style.configure('Stats.Treeview', background='#313244', foreground='#CDD6F4',

fieldbackground='#313244', rowheight=24)

style.configure('Stats.Treeview.Heading', background='#45475A',

foreground='#CDD6F4', font=('Helvetica',9,'bold'))

tree = ttk.Treeview(self.stats_frame, columns=cols, show='headings',

height=12, style='Stats.Treeview')

for c in cols:

tree.heading(c, text=c)

tree.column(c, width=90, anchor='center')

for p in sched.processes:

avg_f = sum(p.frequency_used)/len(p.frequency_used) if p.frequency_used else 0

tree.insert('', 'end', values=(

p.pid, p.name, p.arrival, p.burst,

p.finish_time if p.finish_time else 'N/A',

max(0, p.waiting_time), p.turnaround_time,

f'{p.energy_consumed:.3f}', f'{avg_f:.2f}'

))

tree.pack(fill='both', expand=True, padx=8, pady=4)

# ─────────────────────────────────────────────

if __name__ == '__main__':

app = App()

app.mainloop()